The Bootleg Rock Show

Technical Rider and show info as of Dec 11th 2023

Page 1 of 4

Show Description

The show has 3 x LCD beamers on 1 metre Unicol drop poles. These have a total flown weight of 50K, flown from a downstage bar ( see attached hanging plot sheet ) This projects onto a rear flown white gauze .

We require a projection distance of 5m from the beamers to the gauze. We also have two bars for white “legs “. One end of each bar is clamped onto the same bar as the flown white gauze, the other end being clamped onto a second bar approx. 1m downstage of the gauze bar. We will supply scaff clamps

Lights

We would like to use the house lighting to provide us with a four-colour wash and to haave access to any intelllegent or moving light on your grid.

We also require 4 x PCs of yours on the bridge to focus on our backline and  7 x specials on your front truss to focus on the front musicians.

We will supply a floor package, lighting desk and engineer.

The Show

The stage is then set up as for a regular band lineup.

We project videos behind the band throughout the show

Special FX

We would like to use strobe effect lighting, haze, smoke and a small air-cooled laser

WE WOULD LIKE TO FIRE 2 x MAGIC FX STREAMER CANNONS. THESE ARE POWERED BY COMPRESSED AIR AND ARE NOT PYROTECHNICS. THESE ARE FIRED ONCE, TOWARDS THE END OF THE SHOW (PLEASE ALSO NOTE…THEY ARE STREAMERS,  NOT CONFETTI)

WE WOULD ALSO LIKE TO FIRE 6 X LEMAITRE REDUCED-HEIGHT SILVER JETS AT ONE POINT IN THE SHOW. THESE AND THE STREAMERS WILL BE OPERATED FROM STAGE LEFT BY ONE OF OUR CREW ( QUALIFIED ) RISK ASSESSMENTS WILL BE PROVIDED FOR THE SILVER JETS AND STREAMERS.

Page 2

Control Requirements

We will need a FOH control area of a minimum 6 seats wide 2 seats deep

for sound & lighting desk

We will control onstage monitors & AV from stage left

Tour Party

The band is made up of 6 male singers / musicians and 1 female singer

We will be travelling with 4 crew :

FOH Engineer / Production Manager

Stage Manager / AV Operator

Monitor Engineer

Lighting Engineer

Technical & Staging

We require you to supply 5 x  risers

2 x 8’ x 4’ units at 18” high for drums  ( angled stage left  )

2 x 8’ x 4’ units at 18” high for keyboards ( angled stage right )

1 x 8’ x 4’ unit at 12” high for centre vocals ( note – only 2 x back legs required )

We will require black valance to cover the front of all three risers

The stage / performing area to be covered with black Marley flooring

We require stage size to be 11 metres wide

We require you to supply 4 talk back /comms headsets :

2 stage left,  2 front of house

Sound

We require you to supply a quality full range auditorium speaker system producing full range 20 - 20k, capable of producing 95db at your FOH position

Can you  please provide 4 wedges & amps

2 x to be used as side fills  (in stereo please ) on stands if required

The send to these will be from the monitor board at stage left

2 x to be put at the corners of the stage as audience fills. These can be in mono.

All sends to Main sound system and front audience fills need to be from FOH position

Page 3

We will supply FOH & Monitor desks, mics, leads, mic stands, and all backline and instruments for the show. We have the facility to connect our FOH  desk with rj45 (supplied)

We will be using 3 Radio packs ( for vocals) 3 radio packs ( for guitars )

Frequencies from :

830.100

830.500

831.300

831.900

Plus 1 x tbc

We will also be using 5 In - Ear Monitoring systems

Frequencies from :

863.250         863.100

863.740         863.900

864.625         864.500

864.990         864.900

Power

We require 3Phase 32-amp supply ( at stage left please )

Dressing Rooms

4 rooms are required, please

3 rooms for the artist

1 room on stage level for production

Get In

Load – in time is 1.00 pm ( 13.00 )

Please note we need 4 stage hands for the get in, then 4 for the get out.

IT IS CRUCIAL TO THE SHOW THAT THESE STAGE HANDS ASSIST US WITH DE-RIGGING AND LOADING OUT OUR EQUIPMENT BEFORE THEY DE-RIG ANY HOUSE EQUIPMENT. IF THIS IS NOT POSSIBLE, THEN 2 SEPARATE STAGEHANDS NEED TO BE PROVIDED TO HELP US. WE MUST NOT BE LEFT TO LOAD THE VANS ALONE

During the build we need a flyman, lighting man & sound tech

Build time for the show is 3pprox.. 4 hours

WE HAVE GIVEN YOU TIMINGS FOR A 7.30 pm SHOW. IF THE SHOW IS AT ANY OTHER TIME WE WILL CONTACT YOU AND CHANGE THE ARRANGEMENTS ACCORDINGLY

Page 4

Daily production schedule would therefore be :

1 pm – 4pm get in and build

2 pm  - backline and band arrive

5.00 pm – 6.00 pm sound check

Then final focus / stage blackout until till doors open

The show runs :

65 minutes first half

65 minutes second half

Merchandise

We will sell merchandise before and after the show

We would like an area a minimum of 3m wide, with adequate lighting and mains electricity within 3m of the selling position.

We will also need a selling table.

Parking

We will travel in :	 1 x Mercedes jumbo Sprinter van

1 x  Mercedes Sprinter splitter van

Contacts

Stage Manager

Tony Henderson

Mb 0044 (0) 7885 093379

Email :      tony@transmedia742.net

Production Manager

Arthur Anderson

Mb 0044 (0) 7885 162122

Email:      arthur@transmedia742.net

Website    www.thebootlegrockshow.com